design a system to control an led light using arduino and a mobile app

You will be able to turn ON/OFF an LED using a mobile app that communicates with an Arduino board through Bluetooth.

## 🔧 Components Required:

- Arduino Uno (or any other compatible board)

- Bluetooth module (HC-05 or HC-06)

- LED

- 220Ω resistor

- Jumper wires

- Breadboard

- Smartphone

- Mobile App (like Bluetooth Terminal or custom app)

## ⚙️ Circuit Diagram:

## Arduino Pin      | Connects To

## -----------------|-------------------------

## D13              | LED +ve (through 220Ω resistor)

## GND              | LED -ve

## TX (Pin 1)       | RX of HC-05

## RX (Pin 0)       | TX of HC-05

## 5V               | VCC of HC-05

## GND              | GND of HC-05

Note: Use voltage divider for HC-05 TX to Arduino RX to avoid 5V damage (optional but recommended).

## 🧠 Arduino Code:

## char data = 0;

## void setup() {

## pinMode(13, OUTPUT);   // LED connected to digital pin 13

## Serial.begin(9600);    // Start serial communication at 9600 baud rate

## }

## void loop() {

## if (Serial.available()) {

## data = Serial.read();   // Read the incoming data

## if (data == '1') {

## digitalWrite(13, HIGH); // Turn LED ON

## } else if (data == '0') {

## digitalWrite(13, LOW);  // Turn LED OFF

## }

## }

## }

## 📱 Mobile App :

- App: "Bluetooth Terminal" or "Serial Bluetooth Terminal" (Android)

- How to Use:

  - Pair your phone with HC-05.

  - Open the app, connect to HC-05.

  - Send 1 to turn ON LED, 0 to turn it OFF.

## 💡 Working:

- Power Arduino and connect HC-05.

- Pair your phone with HC-05 using password 1234 or 0000.

- Open Bluetooth Terminal.

- Send 1 to turn ON the LED, 0 to turn it OFF.

## 🔁 Extensions You Can Try:

- Use voice control using Google Assistant + IFTTT

- Add a brightness control using PWM

- Use Wi-Fi (ESP8266 or ESP32) instead of Bluetooth for remote access